# Hierarchical Vision-Language Navigation for an Autonomous Drone Swarm

Project Black Dragon — architecture and simulation plan

Kamalanathan Jude Ernest Paul  ·  7 October 2026  ·  Draft v1

# 1. Summary

This project builds an autonomous drone navigation system in which a small multimodal language model (SLM/VLM) acts as a high-level planner and a reinforcement learning policy acts as the low-level controller. A user issues a natural-language command; the planner decomposes it into an ordered sequence of subgoals using the drone's camera view and current pose; the RL controller executes each subgoal against live LiDAR and pose data.

The system is developed and validated entirely in simulation. Hardware deployment is deliberately deferred until the simulated multi-drone demo is working, at which point the ROS2 node graph transfers unchanged — only the sensor and actuator topics are swapped from simulated to real.

The contribution is the hierarchy itself: a cheap, separately-trained two-tier system, rather than a single monolithic navigation model. Recent releases such as Qwen-RobotNav (Qwen3-VL based, 2B/4B/8B) and Mistral's Robostral Navigate (8B, monocular RGB) occupy the end-to-end navigation space, so positioning should emphasise the decoupled planner/controller design and its training economics.

# 2. Architecture

Four cooperating ROS2 nodes per drone, plus a swarm coordination layer.

## 2.1 Component responsibilities

| Component | Role | Inputs | Outputs |
| --- | --- | --- | --- |
| Mission command | Parses the user's natural-language order | Text command | Mission goal |
| VLM planner | Decomposes the goal into ordered subgoals; detects arrival | RGB camera, pose, command | Structured subgoal |
| RL controller | Executes the active subgoal; avoids obstacles | LiDAR, pose, subgoal | Velocity commands |
| Flight control | Low-level stabilisation and actuation | Velocity commands | Motor commands (PX4) |
| Swarm coordination | Task handoff and deconfliction across drones | Neighbour states | Task assignments |

## 2.2 Why there is no separate perception model

Semantic perception and geometric perception are split rather than merged. The VLM handles semantics — recognising that a target storefront is ahead and to the left — directly from the camera feed, so no separate object detector is needed for that role. Geometric perception stays with the RL controller, which consumes raw LiDAR and pose only.

The consequence is that the RL controller never needs to understand what a "cloth shop" is. It only needs to satisfy a subgoal while not colliding. This keeps the policy small, fast, and trainable on a single consumer GPU, which is the main practical benefit of the split.

## 2.3 The two-rate control loop

The planner and the controller cannot run in lockstep. A 2B-parameter VLM delivers roughly 1–3 inferences per second; the drone control loop needs 20–50 Hz. The planner therefore issues a subgoal that persists while the controller runs hundreds of steps against it.

Replanning is event-driven rather than purely periodic. Triggers:

- Subgoal reached — the controller reports the target state satisfied

- Subgoal failed — the controller cannot make progress toward it

- Stuck detector — displacement below threshold over N control steps

- Timeout — a fixed backstop so the system cannot stall silently

Designing these triggers well is the main engineering risk in the hierarchy. Time-based replanning alone is both wasteful and laggy.

## 2.4 The planner–controller interface

A bare verb such as "go forward" is too low-bandwidth: it discards distance, heading and intent, so the controller cannot distinguish a two-metre clearance manoeuvre from a thirty-metre traversal. The interface is instead a structured subgoal containing a relative waypoint or heading, a semantic target label, and a termination condition.

The planner emits this as JSON under constrained decoding, so malformed or unparseable actions are structurally impossible. A fixed schema also gives the RL controller a finite, well-defined goal space to condition on.

# 3. Training strategy

The controller is trained without the VLM in the loop. Running a VLM inside the RL training loop would slow the simulator by more than an order of magnitude and make convergence impractical on available hardware.

- Train the controller as a goal-conditioned policy (PPO or SAC) on randomly sampled subgoals drawn from the schema. The policy learns: given this subgoal and this LiDAR scan, act.

- Validate the controller in isolation against scripted subgoal sequences.

- Attach the VLM planner at inference time only, and measure end-to-end task success.

- Optionally, run a second-stage reinforcement fine-tuning pass on the planner to adapt it to the controller's behaviour. Prior work on small-VLM navigation planners reports measurable gains from this stage.

# 4. Simulation environment

## 4.1 Stack

Simulator: Gazebo (Harmonic) — chosen over Unreal/AirSim for lower resource cost, native ROS2 integration, and straightforward multi-robot spawning

Middleware: ROS2 (Humble or Jazzy)

Flight stack: PX4 SITL via MAVROS

Host: WSL2 on Windows with GPU passthrough

GPU: NVIDIA GTX 1650 Ti (4 GB VRAM)

## 4.2 Asset pipeline

Scene assets are authored in Blender and imported into Gazebo. Export via glTF/GLB, not Collada: Blender marked Collada I/O legacy in 4.2 and removed it in 5.0, and the robotics ecosystem has converged on glTF/GLB. Gazebo supports glTF natively, embeds textures in a single file, and supports PBR materials.

- Export GLB from Blender; reference it from an SDF <mesh><uri> entry

- The gazebo_sdf_static_model_exporter Blender extension (Blender 5.0+, Gazebo Harmonic+) automates single-click SDF export for static models

- Mark buildings <static>true</static> with simple box collision geometry — visual fidelity matters, per-polygon collision does not

- Invest in textures over geometry: a low-poly facade with a photographic texture and legible signage grounds far better than high-poly untextured geometry

## 4.3 Known constraint: visual domain gap

Gazebo's renderer is not photorealistic, and the VLM is pretrained on real photographs. Grounding accuracy may degrade purely from this gap. Mitigation is to test the chosen VLM on a handful of rendered frames before committing to building the full environment, rather than discovering the problem after weeks of asset work.

# 5. Model selection

The binding constraint is 4 GB of VRAM, not the available model landscape. Candidates, all 4-bit quantised (roughly 1.5–2.5 GB resident):

- Qwen-RobotNav 2B — navigation-specific, Qwen3-VL backbone, familiar tooling

- Qwen2.5-VL 3B — general-purpose, strong grounding

- SmolVLM2 — smallest footprint, lowest capability ceiling

- Moondream — compact, well-suited to single-image queries

The VLM and the RL training run must not occupy the GPU simultaneously. Sequence the work: train the controller first, then load the VLM for inference-time evaluation.

# 6. Roadmap

| Phase | Deliverable | Exit criterion |
| --- | --- | --- |
| 1 | Gazebo + PX4 SITL + ROS2 running; one drone flying | Drone responds to manual MAVROS commands |
| 2 | Full node graph wired with placeholder models | Topic chain runs end to end with dummy logic |
| 3 | Blender-authored semantic world imported | VLM grounds targets in rendered frames above baseline |
| 4 | Goal-conditioned RL controller trained | Controller satisfies scripted subgoals reliably |
| 5 | VLM planner attached at inference | End-to-end command completion measured |
| 6 | Multi-drone swarm (2–3 agents) | Task handoff and deconfliction demonstrated |
| 7 | Recorded simulation demo | Proof of concept suitable for review |
| 8 | Hardware port | Deferred until phase 7 is complete |

# 7. Open questions

- Arrival detection. How does the system verify it has reached the target? This falls to the VLM at each replan step, and a small VLM confidently misidentifying a target is the most probable failure mode in the design. It should be measured explicitly rather than assumed away.

- Replan trigger tuning. The balance between responsiveness and wasted inference has not been characterised.

- Swarm coordination mechanism. Whether to use decentralised multi-agent RL, a graph neural network, or a simpler auction/consensus scheme remains open. The multi-agent RL framing is the stronger research angle; the auction scheme is the lower-risk engineering choice.

- Subgoal schema granularity. Too coarse and the controller lacks direction; too fine and the planner becomes the bottleneck.